RISC-V Processor

Complete RTL to GDS Journey

From Verilog Code to Physical Chip Layout

B.Tech Final Year Project | SkyWater 130nm PDK | OpenLane v1.0.2

March 2026 | Harshith

| Phase 1 — RTL+Sim | Phase 2 — ASIC Flow | Final Result |
| --- | --- | --- |
| 9 Verilog modules 13-instruction test 70.9 MHz, 0 violations IEEE Paper written | OpenLane 42 steps 4 runs (3 errors fixed) 41,448 standard cells 0.81 mm² chip area | GDS tape-out ready DRC = 0 violations LVS = CLEAN IR Drop = 35mV OK |

# PHASE 1 — RTL Design & Simulation

## Step 1: Architecture Design

Before writing any code, the processor architecture was decided:

| Decision | Choice & Reason |
| --- | --- |
| ISA | RISC-V RV32I — open standard, industry-relevant, 32-bit base integer ISA |
| Architecture | Single-cycle — every instruction completes in exactly 1 clock cycle. Simpler than pipelined but slower. |
| Memory | 256×32-bit instruction ROM + 256×32-bit data RAM (separate Harvard architecture) |
| Registers | 32×32-bit register file. x0 hardwired to 0 as per RISC-V spec |
| Tool | Vivado (Xilinx) for RTL design, simulation and timing analysis |

## Step 2: Writing the 9 Verilog Modules

The processor was split into 9 separate modules, each handling one function:

| Module | File | Function |
| --- | --- | --- |
| Top Module | top.v | Connects all modules together |
| Instruction Fetch | instruction_fetch_unit.v | PC register, PC+4, PCSrc mux |
| Instruction Memory | instruction_memory.v | 256×32 ROM, loads program.hex |
| Control Unit | control_unit.v | Decodes opcode → control signals |
| Data Path | data_path.v | Connects RF, ALU, DMEM, muxes |
| Immediate Generator | imm_gen.v | Extracts I/S/B/U/J immediates |
| Register File | register_file.v | 32×32-bit, x0 hardwired, sync write |
| ALU | alu.v | 10 operations, 4-bit ALUControl |
| Data Memory | data_memory.v | 256×32 RAM, async read, sync write |

All 9 files were created using a Python script (create_files.py) because directly typing Verilog in nano was error-prone. The script wrote all files in one go to ~/OpenLane/designs/riscv/src/

## Step 3: Writing the Test Program (program.hex)

A 12-instruction test program was written in machine code (hex format) to verify all major operations:

| Hex | Instruction | Operation | What it tests |
| --- | --- | --- | --- |
| 00500093 | ADDI x1, x0, 5 | x1 = 5 | I-type immediate arithmetic |
| 00300113 | ADDI x2, x0, 3 | x2 = 3 | Second immediate load |
| 002081b3 | ADD x3, x1, x2 | x3 = 8 | R-type register addition |
| 00302023 | SW x3, 0(x0) | mem[0] = 8 | S-type store to memory |
| 00002203 | LW x4, 0(x0) | x4 = 8 | I-type load from memory |
| 00418463 | BEQ x3, x4, +8 | branch if equal | B-type branch (taken, skips NOP) |
| 00000013 | NOP (skipped) | — | Branch target verification |
| 02a00293 | ADDI x5, x0, 42 | x5 = 42 | Load large immediate |
| 40208333 | SUB x6, x1, x2 | x6 = 2 | R-type subtraction (funct7=1) |
| 0020f3b3 | AND x7, x1, x2 | x7 = 1 | R-type bitwise AND |
| 0020e433 | OR x8, x1, x2 | x8 = 7 | R-type bitwise OR |
| 0000006f | JAL x0, 0 | infinite loop | J-type jump instruction |

## Step 4: Simulation in Vivado

Tool used: Vivado XSim (Xilinx Simulation)

A testbench was written that:

- Applied reset for 2 clock cycles

- Ran the clock for 20 cycles (enough for all 12 instructions + NOPs)

- Monitored pc, instruction, alu_result, PCSrc signals on waveform

Results: All 13 test cases PASSED. Key observations from waveform:

- PC increments by 4 each cycle: 0x00 → 0x04 → 0x08 → ...

- BEQ instruction: PCSrc went HIGH → branch taken → PC jumped from 0x14 to 0x1C (skipped NOP)

- JAL at end: PC stays at 0x2C (infinite loop as expected)

## Step 5: Timing Analysis in Vivado

After simulation, timing was analyzed using Vivado's implementation tool:

| Timing Metric | Value | Status |
| --- | --- | --- |
| Clock Period | 14.1 ns | Set as constraint |
| Max Frequency | 70.9 MHz | GOOD |
| Worst Negative Slack (WNS) | +0.098 ns | MET (>0) |
| Total Negative Slack (TNS) | 0.0 ns | MET |
| Setup Violations | 0 | PERFECT |
| Hold Violations | 0 | PERFECT |
| Critical Path | 14.002 ns | IMEM→CU→ALU→DMEM→RF |

## Step 6: IEEE Paper

A full IEEE-format research paper was written documenting the design, simulation results, and timing analysis. The paper included RTL schematic, XSim waveform, and timing summary figures.

File: riscv_ieee_paper_final.docx

# PHASE 2 — ASIC Physical Design (OpenLane RTL-to-GDS)

Tool: OpenLane v1.0.2 (open-source RTL-to-GDS flow by efabless)

PDK: SkyWater 130nm (sky130A) — real manufacturable process

Environment: Docker container on Ubuntu Linux

## What is OpenLane?

OpenLane is an automated RTL-to-GDS flow that runs 42 steps to convert Verilog code into a physical chip layout (GDS file) ready for fabrication. It uses a collection of open-source EDA tools:

| Tool | Step | Purpose |
| --- | --- | --- |
| Yosys | Synthesis | Converts Verilog RTL to gate-level netlist using sky130 cells |
| OpenROAD | Floorplan→Route | Floorplanning, placement, CTS, routing, STA, IR drop |
| Magic | DRC/LVS/GDS | Design Rule Check, Layout vs Schematic, GDS generation |
| KLayout | GDS verify | GDS viewing and XOR verification |
| Netgen | LVS | Netlist comparison between GDS and schematic |

## Setup: Files Created Before Running OpenLane

Before running, these files were prepared:

| File | Purpose |
| --- | --- |
| src/top.v + 8 others | All 9 Verilog modules in ~/OpenLane/designs/riscv/src/ |
| src/program.hex | 12-instruction test program loaded into instruction memory |
| config.json | Tells OpenLane: design name, Verilog files, clock, area, PDN settings |

Important Note: top.v initially had NO output ports (only clk and rst inputs). This caused a critical error in Run 2 — see below.

# The 4 OpenLane Runs — Errors and Fixes

It took 4 attempts to get a successful flow. Each failed run taught us something important about physical design. This is normal in real chip design — engineers rarely get it right on the first try.

## Run 1 — first_run  ❌  Failed at Step 6

| Attribute | Details |
| --- | --- |
| Command | ./flow.tcl -design riscv -tag first_run |
| Failed at | Step 6 — Floorplan (PDN generation) |
| Error | [ERROR PDN-0175] Pitch 2.7600 is too small, must be at least 6.6000 |

### Root Cause

The config.json had no die area specified so OpenLane used a tiny default size. The power grid (PDN) needs a minimum pitch of 6.6 μm between rails, but the chip was too small to fit even that.

### Fix Applied

Added explicit die area and PDN pitch settings to config.json:

"FP_SIZING": "absolute",

"DIE_AREA": "0 0 900 900",

"CORE_AREA": "10 10 890 890",

"FP_PDN_VPITCH": 100,

"FP_PDN_HPITCH": 100,

"FP_PDN_VOFFSET": 10,

"FP_PDN_HOFFSET": 10

Explanation: We gave the chip a fixed 900×900 μm die area (0.81 mm²) and set PDN pitch to 100 μm — well above the 6.6 μm minimum.

## Run 2 — second_run  ❌  Failed at Step 15

| Attribute | Details |
| --- | --- |
| Command | ./flow.tcl -design riscv -tag second_run |
| Failed at | Step 15 — Global Routing |
| Error | [ERROR RSZ-0005] Run global_route before estimating parasitics. Routed nets: 0. Total wirelength: 0 um |

### Root Cause — Logic Trimming

This was the trickiest bug. The original top.v module had ONLY two ports:

module top (input clk, input rst);

The synthesis tool (Yosys) saw: 'This module has no outputs — nothing driven out. I will optimize away all the logic since it has no observable effect.'

This is called LOGIC TRIMMING — the synthesizer legally removes all internal logic because it cannot affect any output. Result: 0 cells placed, 0 nets routed.

### Fix Applied

Added 6 output ports to top.v to force the synthesizer to keep all logic:

module top (

input  clk, rst,

output [31:0] pc_out,        // forces IFU to stay

output [31:0] alu_result_out, // forces ALU to stay

output        reg_write_out,  // forces CU to stay

output        mem_write_out,  // forces DMEM to stay

output        branch_out,     // forces branch logic

output        pcsrc_out       // forces PCSrc mux

);

Now Yosys could see: 'There are outputs — I must keep all logic that drives them.'

## Run 3 — third_run  ❌  Failed at Step 7

| Attribute | Details |
| --- | --- |
| Command | ./flow.tcl -design riscv -tag third_run |
| Failed at | Step 7 — Global Placement |
| Error | [ERROR GPL-0302] Use a higher -density. Given: 0.35. Suggested: 0.41 |

### Root Cause

Now the design had real logic (41,000+ cells), but the target density was set too low at 0.35 (35%). OpenLane's placer calculated that the actual minimum needed density was 0.41 (41%). With density too low, cells would be too spread out and routing would fail.

### Fix Applied

OpenLane's error message actually told us exactly what to set:

"FP_CORE_UTIL": 45,

"PL_TARGET_DENSITY": 0.50

Raised target density from 0.35 to 0.50 (50%) — well above the suggested 0.41.

## Run 4 — fourth_run  ✓  SUCCESS!

| Attribute | Details |
| --- | --- |
| Command | ./flow.tcl -design riscv -tag fourth_run |
| Status | [SUCCESS]: Flow complete. All 42 steps passed! |
| Runtime | 27 minutes 57 seconds |
| DRC | 0 violations — PERFECT |
| LVS | CLEAN — 41,452 nets matched |

# The 42 OpenLane Steps — What Each Step Does

| # | Step Name | What it does |
| --- | --- | --- |
| 1 | Synthesis (Yosys) | Converts your Verilog RTL to a gate-level netlist using sky130 standard cells (AND, OR, FF etc). Result: top.v with real gates. |
| 2 | Pre-layout STA | Static Timing Analysis on the synthesized netlist before placement. Checks setup/hold timing. |
| 3 | Floorplan Init | Sets the die boundary and core area. Determines where cells will be placed. |
| 4 | IO Placement | Places IO pads (clk, rst, output pins) around the chip boundary. |
| 5 | Tap/Decap Insert | Inserts 11,050 tapcells (latch-up prevention) and decap cells (power noise reduction). |
| 6 | PDN Generation | Creates the Power Distribution Network — metal grid of VPWR/VGND rails on met3/met4/met5. |
| 7 | Global Placement | Places all 41,448 standard cells roughly in the core area. Optimizes for wirelength. |
| 8-11 | Resize + Opt | Resizes cells for timing, fixes hold violations, buffers long nets, inserts repeaters. |
| 12 | Clock Tree Synthesis | Inserts 824 clkbuf_8 buffers in 6 levels to distribute clock to all 8,736 sinks with low skew. |
| 13-14 | Post-CTS STA | Re-runs timing analysis after clock tree insertion. Hold fixing if needed. |
| 15-18 | Detail Placement | Legalizes cell placement — ensures no overlaps, cells snap to grid rows. |
| 19-22 | Global Routing | Plans routes between cells across the chip using a grid-based coarse routing. |
| 23 | Detailed Routing | Actual metal wire routing — connects every cell pin with real wires on met1-met5 layers. Longest step (~15 min). |
| 24-30 | SPEF + Multi-STA | Extracts actual RC parasitics from routed wires. Runs STA at 3 corners: min, max, nominal. |
| 31 | Single Corner STA | Final timing signoff at typical corner (TT 25°C 1.80V). WNS=0.0ns, TNS=0.0ns. |
| 32 | IR Drop Analysis | Checks voltage drop across PDN. VPWR worst: 35.3mV (1.96%). VGND worst: 34.5mV. Both PASSED. |
| 33-34 | GDS Generation | Magic and KLayout generate the final GDSII chip layout file (top.gds). |
| 35 | XOR Check | Compares Magic GDS vs KLayout GDS using XOR. Result: 0 differences. |
| 36-38 | Netlist Export | Exports SPICE netlist, powered Verilog, and final gate-level Verilog. |
| 39 | LVS | Layout vs Schematic — verifies GDS matches the netlist. Result: CLEAN, 41,452 nets matched. |
| 40 | Magic DRC | Design Rule Check — verifies all manufacturing rules obeyed. Result: COUNT=0 (perfect). |
| 41 | Antenna Check | Checks for antenna violations. 91 antenna diodes were inserted to fix violations. |
| 42 | ERC | Electrical Rule Check — final electrical verification. Flow complete! |

# Final Results Summary

## Physical Design Metrics

| Metric | Value |
| --- | --- |
| Die Area | 0.81 mm² (900 × 900 μm) |
| Total Standard Cells | 101,935 |
| Logic Cells | 41,448 |
| Fill Cells | 49,346 |
| Tapcells | 11,050 |
| Antenna Diodes | 91 |
| Flip-Flops (DFF) | 30 |
| Total Wire Length | 702,868 μm (702 mm) |
| Cell Density | 51,170 cells/mm² |

## Timing & Quality

| Metric | Value | Status |
| --- | --- | --- |
| Clock Period | 14.1 ns | — |
| Maximum Frequency | 70.9 MHz | GOOD |
| Worst Negative Slack (WNS) | 0.0 ns | MET ✓ |
| Total Negative Slack (TNS) | 0.0 ns | MET ✓ |
| Hold Slack (worst) | 0.35 ns | MET ✓ |
| DRC Violations | 0 | PERFECT ✓ |
| LVS | CLEAN | PERFECT ✓ |
| IR Drop (VPWR worst) | 35.3 mV | PASSED ✓ |
| XOR Differences (Magic vs KLayout) | 0 | PERFECT ✓ |

## CTS Results

| CTS Metric | Value |
| --- | --- |
| Clock Net | clk |
| Total Clock Sinks | 8,736 |
| Buffers Inserted | 824 (sky130_fd_sc_hd__clkbuf_8) |
| Clock Tree Levels | 6 |
| Min/Max buffers per path | 3 / 4 (only 1 level difference) |
| Max Fanout per buffer | 15 sinks |

## Output Files Generated

| File | Description |
| --- | --- |
| results/final/gds/top.gds | FINAL CHIP LAYOUT — tape-out ready GDS file |
| results/synthesis/top.v | Gate-level netlist with real sky130 cells |
| results/synthesis/top.sdf | Standard Delay Format — post-synthesis timing |
| results/floorplan/top.def | Floorplan DEF file |
| results/placement/top.def | Placed cells DEF file |
| results/routing/top.def | Routed design DEF file |
| reports/manufacturability.rpt | DRC=0, LVS=CLEAN, Antenna summary |
| logs/signoff/32-irdrop.log | IR drop analysis: VPWR 35.3mV, VGND 34.5mV |

# Tools Used — Complete Reference

| Tool | Phase | What we used it for |
| --- | --- | --- |
| Vivado | Phase 1 | RTL design, XSim simulation, waveform viewing, timing analysis |
| Docker | Phase 2 | Container environment — all OpenLane tools run inside Docker for reproducibility |
| OpenLane | Phase 2 | RTL-to-GDS automation flow — runs all 42 steps |
| Yosys | Phase 2 | Logic synthesis. Also used separately to generate schematic.dot via Yosys show command |
| OpenROAD | Phase 2 | Floorplan, placement, CTS, routing, STA, IR drop. GUI used to view each stage |
| Magic | Phase 2 | DRC, LVS, GDS streaming inside OpenLane flow |
| KLayout 0.29.0 | Phase 2 | Viewing final GDS layout. Installed from klayout.org .deb package |
| Graphviz (dot) | Phase 2 | Converted Yosys .dot schematic file to .png image |
| nano | Both | Terminal text editor for editing config.json and Verilog files |
| Python 3 | Both | Script to create all 9 Verilog files in one shot (create_files.py) |
| SkyWater 130nm PDK | Phase 2 | Process Design Kit — contains sky130_fd_sc_hd standard cell library with timing/power models |

# Key Viva Q&A — Explain to Your Guide

## Q: What is the overall project in one sentence?

"I designed a 32-bit RISC-V single-cycle processor in Verilog, verified it at 70.9 MHz in Vivado simulation, then ran the complete RTL-to-GDS physical design flow using OpenLane and SkyWater 130nm PDK — resulting in a tape-out ready chip layout with 0 DRC violations."

## Q: Why did first_run fail?

The default chip area was too small for the power grid. OpenLane requires a minimum PDN pitch of 6.6 μm but the auto-sized chip was too tiny. Fix: Added explicit die area of 900×900 μm and set PDN pitch to 100 μm.

## Q: Why did second_run fail?

Logic trimming — the original top.v had no output ports, so the synthesizer (Yosys) removed all internal logic since it had no observable effect on any output. Fix: Added 6 output ports (pc_out, alu_result_out etc.) to force Yosys to keep all logic.

## Q: Why did third_run fail?

Placement density too low. The target density was 0.35 but OpenLane calculated the design needed at least 0.41. Fix: Raised target density to 0.50.

## Q: What is CTS and what did it do in your design?

Clock Tree Synthesis inserts clock buffers to distribute the clock signal to every flip-flop at the same time, minimizing clock skew. In our design, OpenROAD inserted 824 clkbuf_8 buffers in a 6-level balanced tree to reach 8,736 clock sinks. The max path difference was only 1 buffer level, ensuring very low skew.

## Q: What is IR drop? Did your design pass?

IR drop is the voltage drop across the resistance of power rails (V=IR). Our design had worst-case VPWR IR drop of 35.3 mV — only 1.96% of the 1.8V supply, well below the 10% budget of 180 mV. Both VPWR and VGND passed.

## Q: What is DRC and what was your result?

Design Rule Check verifies that your chip layout follows all manufacturing rules — minimum wire widths, spacing between wires, via sizes etc. Our result: 0 DRC violations — perfect, tape-out quality.

## Q: What is LVS?

Layout vs Schematic — verifies that the physical GDS layout matches the logical netlist. Our result: CLEAN, 41,452 nets matched. This means the chip will behave exactly as designed.

## Q: Why did you use OpenLane instead of commercial tools?

OpenLane is the only open-source complete RTL-to-GDS flow available for students. It uses the SkyWater 130nm PDK which is a real manufacturable process (Google and SkyWater offer free MPW shuttles). Commercial alternatives like Cadence Innovus or Synopsys IC Compiler cost millions of dollars.

RISC-V Processor Project | B.Tech Final Year | SkyWater 130nm | OpenLane v1.0.2 | March 2026