RISC-V Single-Cycle Processor

Power Planning & IR Drop Analysis Report

PDN Analysis | SkyWater 130nm | OpenROAD PSM

OpenLane v1.0.2 | fourth_run | March 2026

# 1. Power Planning Overview

Power planning is a critical step in physical design that ensures reliable power delivery to all standard cells across the chip. It involves creating a Power Distribution Network (PDN) — a grid of metal stripes and rails that deliver VDD (VPWR) and GND (VGND) to every logic gate.

In this design, power planning was performed in Steps 5 and 6 of the OpenLane flow using OpenROAD's PDN generator. The IR drop analysis was performed post-route using the OpenROAD Power Supply Manager (PSM).

| Parameter | VPWR (VDD) | VGND (GND) |
| --- | --- | --- |
| Supply Voltage | 1.800 V | 0.000 V |
| Worstcase Voltage | 1.760 V | 0.0345 V |
| Average IR Drop | 18.9 mV | 18.8 mV |
| Worst IR Drop | 35.3 mV | 34.5 mV |
| PDN Nodes | 22,068 | 21,906 |
| Voltage Sources | 6 | 6 |
| All PDN Stripes Connected | YES ✓ | YES ✓ |
| IR Drop Budget (10% of VDD) | 180 mV | 180 mV |
| IR Drop Status | PASSED ✓ | PASSED ✓ |

# 2. Power Distribution Network (PDN) Structure

The PDN is a hierarchical metal grid that distributes power from the chip boundary to individual standard cells. It consists of multiple metal layers working together:

| Metal Layer | Direction | Pitch (μm) | Function |
| --- | --- | --- | --- |
| met1 (li1) | Horizontal | Standard cell height | Local power rails inside each standard cell row |
| met2 | Vertical | — | Signal routing (power connections via vias) |
| met3 | Horizontal | 100 μm | Horizontal power stripes (VPWR/VGND) |
| met4 | Vertical | 100 μm | Vertical power stripes (VPWR/VGND) |
| met5 | Horizontal | — | Top-level power distribution |

## 2.1 PDN Configuration Parameters

| PDN Parameter | Value |
| --- | --- |
| PDN Vertical Pitch (FP_PDN_VPITCH) | 100 μm |
| PDN Horizontal Pitch (FP_PDN_HPITCH) | 100 μm |
| PDN Vertical Offset (FP_PDN_VOFFSET) | 10 μm |
| PDN Horizontal Offset (FP_PDN_HOFFSET) | 10 μm |
| Power Net | VPWR (VDD = 1.800V) |
| Ground Net | VGND (GND = 0.000V) |
| Core Area | 10,10 → 890,890 μm |
| Die Area | 0,0 → 900,900 μm (0.81 mm²) |
| Metal Node Density | Standard cell height × 5 |
| Bump Pitch X direction | 140 μm (default checkerboard) |
| Bump Pitch Y direction | 140 μm (default checkerboard) |

# 3. IR Drop Analysis

IR drop refers to the voltage drop across the resistance of the power distribution network. Since the metal wires have finite resistance, current flowing through them causes a voltage drop (V = I × R). Excessive IR drop reduces the effective supply voltage at cell inputs, causing timing failures or functional errors.

## 3.1 VPWR (VDD) IR Drop Analysis

| VPWR Metric | Value | Status |
| --- | --- | --- |
| Nominal Supply Voltage | 1.800 V | — |
| Worstcase Voltage at any node | 1.760 V | GOOD |
| Average IR Drop | 18.9 mV (1.05%) | EXCELLENT |
| Worst IR Drop | 35.3 mV (1.96%) | ACCEPTABLE |
| IR Drop Budget (10% VDD) | 180 mV | — |
| Margin (Budget - Worst drop) | 144.7 mV | PASSED ✓ |
| Total PDN Nodes (VPWR) | 22,068 | — |
| All PDN Stripes Connected | YES | PASSED ✓ |

## 3.2 VGND (GND) IR Drop Analysis

| VGND Metric | Value | Status |
| --- | --- | --- |
| Nominal Ground Voltage | 0.000 V | — |
| Worstcase Ground Bounce | 34.5 mV | ACCEPTABLE |
| Average Ground Bounce | 18.8 mV (1.04%) | EXCELLENT |
| Ground Bounce Budget (10% VDD) | 180 mV | — |
| Margin (Budget - Worst bounce) | 145.5 mV | PASSED ✓ |
| Total PDN Nodes (VGND) | 21,906 | — |
| All PDN Stripes Connected | YES | PASSED ✓ |

# 4. Power Planning Special Cells

Special cells are inserted during power planning to ensure robust power delivery and prevent manufacturing issues:

| Cell Type | Count | Library Cell | Purpose |
| --- | --- | --- | --- |
| Tapcells | 11,050 | sky130_fd_sc_hd__tapvpwrvgnd_1 | Prevent latch-up by connecting substrate/well taps to VPWR/VGND at regular intervals |
| Decap cells | ~included | sky130_ef_sc_hd__decap_12 | Decoupling capacitors — reduce power supply noise by providing local charge reservoir |
| Filler cells | 49,346 | sky130_fd_sc_hd__fill_1/2 | Fill empty spaces in rows to maintain N-well continuity and satisfy DRC rules |
| Antenna diodes | 91 | sky130_fd_sc_hd__diode_2 | Protect gates from antenna effect during metal etching process |

# 5. Power Consumption Estimation

Power analysis is performed at the typical process corner (TT 25°C 1.80V):

| Power Component | Value | Description |
| --- | --- | --- |
| Internal Power | 198,866 nW ≈ 0.2 mW | Switching inside cells |
| Switching Power | ~0 nW | Net capacitance switching |
| Leakage Power | ~0 nW | Static transistor leakage |
| Total Estimated Power | ~0.2 mW | At 70.92 MHz, 1.80V |
| Supply Voltage | 1.800 V | Typical corner |
| Estimated Current Draw | ~0.11 mA | P/V = 0.2mW/1.8V |

# 6. IR Drop — Technical Explanation

IR drop analysis verifies that the power supply voltage remains within acceptable limits at every point in the chip. The analysis uses the following formula:

V_drop = I × R_metal

| Parameter | Our Design | Explanation |
| --- | --- | --- |
| Average IR Drop | ~19 mV | Excellent — well below 10% budget of 180mV |
| Worst IR Drop | ~35 mV | 1.96% of VDD — acceptable for prototype design |
| Industry Budget | ≤ 180 mV | 10% of nominal VDD (1.8V × 10% = 180mV) |
| Our Margin | 144.7 mV | Worst drop (35mV) is 80% below the budget limit |

# 7. Power Planning Conclusion

The power distribution network for the RISC-V processor is well-designed and passes all IR drop checks:

✓  All PDN stripes on VPWR are connected — no open power rails

✓  All PDN stripes on VGND are connected — no open ground rails

✓  Average IR drop: 18.9 mV (1.05%) — well within 10% budget

✓  Worst IR drop: 35.3 mV (1.96%) — 80% below budget limit

✓  11,050 tapcells inserted — latch-up prevention ensured

✓  91 antenna diodes inserted — antenna violations minimized

✓  22,068 VPWR nodes / 21,906 VGND nodes fully connected

✓  Total estimated power: ~0.2 mW at 70.92 MHz, 1.80V

The power planning is robust and the chip can safely operate at the target frequency of 70.92 MHz with the SkyWater 130nm process at nominal supply voltage of 1.80V.

Generated by OpenLane v1.0.2 | OpenROAD PSM | SkyWater 130nm PDK | B.Tech Final Year Project