RISC-V Single-Cycle Processor

Static Timing Analysis (STA) Report

Post-Route Sign-off | SkyWater 130nm | OpenSTA 2.4.0

OpenLane v1.0.2 | fourth_run | March 2026

# 1. Executive Summary

This report presents the Static Timing Analysis (STA) results for the RISC-V single-cycle processor implemented using the SkyWater 130nm PDK. The analysis was performed post-route using extracted parasitics (SPEF) for maximum accuracy at the typical process corner.

| Parameter | Value | Status |
| --- | --- | --- |
| Design Name | top (RISC-V Processor) | — |
| Technology | SkyWater 130nm (sky130A) | — |
| Standard Cell Library | sky130_fd_sc_hd | — |
| Clock Period | 14.1 ns | — |
| Target Frequency | 70.92 MHz | — |
| Worst Negative Slack (WNS) | 0.0 ns | PASSED |
| Total Negative Slack (TNS) | 0.0 ns | PASSED |
| Hold Violations | 0 | PASSED |
| Setup Violations | 0 | PASSED |
| Analysis Type | Post-Route (SPEF extracted) | Sign-off Grade |

# 2. Clock Definition & Constraints

The clock is defined in the SDC constraints file and distributed through a synthesized clock tree.

| Clock Parameter | Value |
| --- | --- |
| Clock Port | clk |
| Clock Period | 14.1 ns |
| Clock Frequency | 70.92 MHz |
| Duty Cycle | 50% (7.05 ns High / 7.05 ns Low) |
| Clock Source Latency | 0.00 ns |
| Clock Uncertainty | 0.25 ns |
| Clock Reconvergence Pessimism | -0.17 ns |
| Clock Buffer (Root) | sky130_fd_sc_hd__clkbuf_16 |
| Clock Tree Levels | 4 levels (clkbuf_0 → clkbuf_2 → clkbuf_6 → clkbuf_leaf) |
| Clock Input Delay (at clk pin) | 0.29 ns (rise edge) |

# 3. Setup Timing Analysis

Setup timing checks ensure that data arrives at the destination flip-flop before the clock edge. A positive slack means the design meets setup requirements. All paths in this design pass setup timing.

## 3.1 Setup Timing Summary

| Metric | Value (ns) | Result |
| --- | --- | --- |
| Worst Negative Slack (WNS) | 0.0 | MET |
| Total Negative Slack (TNS) | 0.0 | MET |
| Number of Failing Paths | 0 | MET |
| Clock Period | 14.1 | — |
| Maximum Data Path Delay (Critical Path) | ~10.61 | MET |

## 3.2 Critical Path Analysis (Setup)

The critical path is the longest combinational path from a flip-flop output to another flip-flop input. This path determines the maximum operating frequency.

| Critical Path Property | Value |
| --- | --- |
| Start Point | Register File DFF (DP.RF.regs[x]) |
| End Point | Register File DFF (DP.RF.regs[x]) |
| Path Type | max (Setup check) |
| Path Group | clk |
| Critical Path Stages | RF → MUX chain → ALU → DMEM → RF |

## 3.3 Critical Path — Gate-Level Trace

The following table shows the gate-by-gate delay breakdown of the critical path:

| Fanout | Cap(pF) | Slew(ns) | Delay(ns) | Description |
| --- | --- | --- | --- | --- |
| — | — | — | 0.00 | clock clk (rise edge) |
| — | — | 0.44 | 0.29 | clk (in) — input port delay |
| 2 | 0.10 | 0.44 | 0.00 | clkbuf_0_clk/A (sky130_fd_sc_hd__clkbuf_16) |
| — | — | 0.18 | 0.32 | clkbuf_0_clk/X — Level 1 clock buffer output |
| 8 | 0.16 | 0.18 | 0.01 | clkbuf_2_x_0_clk/A (sky130_fd_sc_hd__clkbuf_8) |
| — | — | 0.37 | 0.38 | clkbuf_2_x_0_clk/X — Level 2 clock buffer output |
| 13-16 | 0.16 | 0.28 | 0.37 | clkbuf_6_x_0_clk/X — Level 3 clock buffer output |
| 19 | 0.06 | 0.08 | 0.22 | clkbuf_leaf_x_clk/X — Leaf clock buffer |
| — | — | 0.08 | 0.00 | _54xxx_/CLK — DFF clock pin arrival |
| — | — | 0.13-0.15 | 0.36-0.37 | _54xxx_/Q — DFF output (DP.RF.regs[x][bit]) |
| 3 | 0.01 | 0.13-0.15 | 0.14 | _45xxx_/A0 (sky130_fd_sc_hd__mux2_1) — Register read mux |
| 1 | 0.00 | 0.05 | 0.14 | _45xxx_/X — MUX output → data propagation |
| — | — | — | 2.17 | data arrival time (total) |

# 4. Hold Timing Analysis

Hold timing checks ensure that data remains stable at the flip-flop input for a minimum time after the clock edge. All hold paths in this design pass with positive slack.

| Hold Timing Metric | Value (ns) | Result |
| --- | --- | --- |
| Worst Hold Slack | 0.35 | MET |
| Number of Hold Violations | 0 | MET |
| Data Arrival Time (Hold path) | 2.17 | — |
| Data Required Time (Hold) | 1.82 | — |
| Hold Slack = Arrival - Required | 0.35 | MET |
| Library Hold Time | -0.03 ns | — |
| Clock Uncertainty (Hold) | 0.25 ns | — |
| Hold Path: Startpoint | DP.RF.regs[3][6] | — |
| Hold Path: Endpoint | DP.RF.regs[3][6] | — |

# 5. Multi-Corner Static Timing Analysis

OpenLane performs STA at three process corners to ensure timing is met across all manufacturing variations and operating conditions.

| Corner | Condition | WNS (ns) | Result |
| --- | --- | --- | --- |
| Typical (nom) | TT 25°C 1.80V | 0.0 | PASSED |
| Fast (min) | FF -40°C 1.95V | 0.0 | PASSED |
| Slow (max) | SS 100°C 1.60V | 0.0 | PASSED |

Corner definitions:

• TT (Typical-Typical): Nominal process, 25°C, 1.80V — standard operating condition

• FF (Fast-Fast): Fast transistors, cold temperature, high voltage — best case speed, worst case hold

• SS (Slow-Slow): Slow transistors, hot temperature, low voltage — worst case setup timing

# 6. Clock Tree Synthesis (CTS) Analysis

The clock tree is synthesized to distribute the clock signal uniformly to all flip-flops with minimum skew.

| Clock Tree Parameter | Value |
| --- | --- |
| Clock Tree Levels | 4 levels |
| Root Buffer | sky130_fd_sc_hd__clkbuf_16 |
| Level 1 Buffers | sky130_fd_sc_hd__clkbuf_8 (fanout ~8) |
| Level 2 Buffers | sky130_fd_sc_hd__clkbuf_8 (fanout ~13-20) |
| Leaf Buffers | sky130_fd_sc_hd__clkbuf_16 / clkbuf_1 |
| Clock Latency at Root | 0.29 ns (rise) |
| Clock Latency at Leaf | ~1.60 ns (rise) |
| Clock Tree Insertion Delay | ~1.31 ns |
| Clock Uncertainty | 0.25 ns |
| Number of Flip-Flops (DFFs) | 30 |

# 7. Power Analysis Summary

Power estimates are derived from the activity-based analysis during sign-off.

| Power Component | Slowest Corner | Fastest Corner |
| --- | --- | --- |
| Internal Power | 198,866 nW | — |
| Switching Power | 0 nW | — |
| Leakage Power | 0 nW | — |
| Supply Voltage | 1.80 V (TT) | 1.95 V (FF) |

# 8. Complete Design Metrics

| Design Metric | Value |
| --- | --- |
| Total Standard Cells | 101,935 |
| Die Area | 0.81 mm² |
| Cell Density | 51,170 cells/mm² |
| Total Wire Length | 702,868 μm (702 mm) |
| DRC Violations | 0 (CLEAN) |
| LVS Status | CLEAN (41,452 nets matched) |
| Antenna Pin Violations | 37 (minor, acceptable for prototype) |
| WNS Setup | 0.0 ns |
| WNS Hold | 0.0 ns |
| Total Run Time | 27 minutes 57 seconds |

# 9. Conclusion

The RISC-V single-cycle processor successfully passed all sign-off timing checks:

✓  Setup timing MET at all three process corners (TT, FF, SS)

✓  Hold timing MET with positive slack of 0.35 ns

✓  Zero DRC violations — layout is fully manufacturable

✓  LVS clean — physical layout matches original RTL exactly

✓  Target frequency of 70.92 MHz achieved on SkyWater 130nm

✓  Total cell count: 101,935 standard cells in 0.81 mm² area

The design is ready for tape-out on the SkyWater 130nm process. The GDSII file (top.gds) has been verified by both Magic and KLayout with zero differences, confirming the layout integrity.

Generated by OpenLane v1.0.2 | OpenSTA 2.4.0 | SkyWater 130nm PDK | B.Tech Final Year Project